ESP32 RESCUE ROBOT — HARDWARE WIRING BLUEPRINT

Complete Connection Reference | Based on Final Firmware

⚠ WARNING: Wire this exactly as listed. Skipping a ground wire or bypassing the voltage divider will destroy the ESP32.

# 1. Power Distribution & Common Ground

⚠ WARNING: You MUST physically join: L298N GND + ESP32 GND + PCA9685 GND + Relay GND + All Sensor GNDs on one rail.

| From | To | Rail / Wire | Notes |
| --- | --- | --- | --- |
| 12V Battery (+) | L298N 12V IN | 12V Power | Main motor power |
| 12V Battery (+) | Relay Module COM | 12V Power | Pump relay supply |
| 12V Battery (+) | PNP Sensor Brown | 12V Power | Sensor Vcc |
| 12V Battery (–) | L298N GND | GND Master | Start ground rail |
| L298N GND | ESP32 GND, PCA9685 GND, Relay GND, All Sensor GNDs | Common GND | ALL grounds tied together |
| L298N 5V OUT | ESP32 VIN | 5V Logic | Logic power source |
| L298N 5V OUT | PCA9685 VCC + V+ | 5V Logic | Servo driver power |
| L298N 5V OUT | HC-SR04 VCC | 5V Logic | Ultrasonic sensor |
| L298N 5V OUT | MQ-2 VCC | 5V Logic | Gas sensor power |
| L298N 5V OUT | Fire Sensor VCC | 5V Logic | Fire sensor power |

# 2. PNP Metal Sensor Voltage Divider (GPIO 32)

⚠ WARNING: The PNP sensor output is 12V. A resistor divider steps it down to 3.3V-safe logic for the ESP32. Do NOT skip this.

| Component / Node | Connect To | Value | Purpose |
| --- | --- | --- | --- |
| PNP Sensor Brown | 12V Battery (+) | 12V | Sensor supply |
| PNP Sensor Blue | Common GND | GND | Sensor return |
| PNP Sensor Black (Signal) | 10kΩ Resistor (end A) | — | Signal line start |
| 10kΩ Resistor (end B) | ESP32 GPIO 32 | 10kΩ | Middle junction |
| ESP32 GPIO 32 | Common GND (via 3.3kΩ) | 3.3kΩ | Ground bleed resistor — protects 3.3V GPIO |

Divider Output Voltage = 12V × 3.3kΩ / (10kΩ + 3.3kΩ) ≈ 2.98V  ✓ (Safe for ESP32 GPIO)

# 3. Fire Detection & Rescue Subsystem

⚡ NOTE: Fire sensor is Active LOW. Relay is Active LOW. Pump activates on fire detection OR V10 manual override from Blynk.

| Component | ESP32 / Target Pin | Logic Level | Notes |
| --- | --- | --- | --- |
| Fire Sensor DO | ESP32 GPIO 33 | Active LOW | LOW = fire detected |
| Relay Module IN | ESP32 GPIO 15 | Active LOW | LOW = pump on |
| Water Pump (+) | Relay NO (Normally Open) | 12V switched | Pump positive terminal |
| Water Pump (–) | 12V Battery (–) / GND | GND | Pump return |
| Metal Sensor Black (via divider) | ESP32 GPIO 32 | Active HIGH (PNP) | HIGH = metal detected |

# 4. Locomotion — L298N Motor Driver

⚡ NOTE: ENA and ENB use ESP32 LEDC PWM (20kHz, 8-bit). Dead-band of ±40 prevents motor chatter at joystick center.

| L298N Pin | ESP32 GPIO | Function | Notes |
| --- | --- | --- | --- |
| IN1 | GPIO 12 | Motor A Dir | Left motor direction 1 |
| IN2 | GPIO 14 | Motor A Dir | Left motor direction 2 |
| IN3 | GPIO 27 | Motor B Dir | Right motor direction 1 |
| IN4 | GPIO 26 | Motor B Dir | Right motor direction 2 |
| ENA | GPIO 13 (PWM) | Motor A Speed | PWM @ 20kHz, 8-bit — Left motor speed |
| ENB | GPIO 25 (PWM) | Motor B Speed | PWM @ 20kHz, 8-bit — Right motor speed |

# 5. Robotic Arm — PCA9685 Servo Driver (I2C)

⚡ NOTE: I2C address is 0x40 (default). SDA = GPIO 21, SCL = GPIO 22. PWM frequency set to 60Hz. Servo pulse range: 150–600.

| PCA9685 Pin | ESP32 GPIO | Protocol | Notes |
| --- | --- | --- | --- |
| SDA | GPIO 21 | I2C Data | Servo arm channel 0–3 |
| SCL | GPIO 22 | I2C Clock | PCA9685 @ 60Hz PWM |
| VCC | 5V Rail (L298N out) | Logic 5V | IC logic supply |
| V+ | 5V Rail (L298N out) | Servo 5V | Servo power supply |
| GND | Common GND | GND | Must share ground with ESP32 |
| CH0 | — | Servo 0 (V2) | Maps Blynk V2 → Arm joint 1 |
| CH1 | — | Servo 1 (V3) | Maps Blynk V3 → Arm joint 2 |
| CH2 | — | Servo 2 (V4) | Maps Blynk V4 → Arm joint 3 |
| CH3 | — | Servo 3 (V5) | Maps Blynk V5 → Arm joint 4 |

# 6. Sensors

⚡ NOTE: GPIO 34 is INPUT ONLY — no internal pull-up available. MQ-2 analog output connects here. Threshold default: 1200.

| Sensor | ESP32 GPIO | Signal Type | Notes |
| --- | --- | --- | --- |
| HC-SR04 TRIG | GPIO 5 | Digital OUT | 10µs pulse to trigger |
| HC-SR04 ECHO | GPIO 18 | Digital IN | Pulse width = distance; 30ms timeout |
| MQ-2 AOUT | GPIO 34 (ADC) | Analog IN | GPIO 34 is input-only; no pull-up needed |

# 7. Blynk Virtual Pin Map

Template ID: TMPL3pysIwKEL  |  Template Name: Robot  |  Update interval: V6=1s, V7=100ms, V8/V9=500ms

| Virtual Pin | Function | Direction | Widget |
| --- | --- | --- | --- |
| V0 | Joystick X (steering) | App → ESP32 | Joystick widget |
| V1 | Joystick Y (throttle) | App → ESP32 | Joystick widget |
| V2 | Servo 0 — Arm Joint 1 | App → ESP32 | Slider 0–180° |
| V3 | Servo 1 — Arm Joint 2 | App → ESP32 | Slider 0–180° |
| V4 | Servo 2 — Arm Joint 3 | App → ESP32 | Slider 0–180° |
| V5 | Servo 3 — Arm Joint 4 | App → ESP32 | Slider 0–180° |
| V6 | MQ-2 Gas Value (0–4095) | ESP32 → App | Gauge / Display |
| V7 | HC-SR04 Distance (cm) | ESP32 → App | Gauge / Display |
| V8 | Metal Detected (0/1) | ESP32 → App | LED indicator |
| V9 | Fire Detected (0/1) | ESP32 → App | LED indicator |
| V10 | Manual Water Pump Override | App → ESP32 | Button / Switch |

All connections verified against firmware (ESP32 + Blynk IoT + Adafruit PCA9685 + L298N + HC-SR04 + MQ-2 + PNP Metal Sensor + Fire Sensor + Relay)